# IMPRESSIONS INVESTIGATION: Shoe print Analysis

Pattern and impression evidence includes any markings produced when one object comes into contact with another object, such as tool marks, tire tracks, and shoe prints.

# LAB ACTIVITY

## Dental Stone Casting - 3D Impression

Dental stone is utilized to cast three-dimensional impressions. It contains gypsum, which allows for a harder and more durable cast than other casting materials.

Materials (per pair of students)

- 2 pounds of dental stone

- Scale

- Plastic ziptop bag

- 8–12 oz of water

- Tongue depressors

- Ruler

- Camera

Procedure

- Photograph the shoe impression from a 90° angle with different flash angles to properly document the evidence.

- Weigh approximately two pounds of dental stone into a plastic bag.

- Add 8–12 oz of water to the bag containing the dental stone.

- Securely seal the bag and mix until the consistency of the dental stone is evenly blended and without clumps. The mixture should resemble pancake batter.

- Pour the dental stone directly into the impression and allow it to sit until cured, approximately 30–45 minutes. You may need to smooth out the mixture with a tongue depressor.

- Once the dental stone has hardened, gently remove and clean the casting.

- Photograph the casting.

## 2D Inked Impression

Inking shoes provides examiners with a known print to compare to shoe prints left at the crime scene.

Materials

- Known shoe

- Black printer’s ink

- Ink roller

- White paper

- Ruler

- Camera

Procedure

- Using an ink roller, roll black ink onto the sole of the shoe until it is covered with a thin, even layer of ink across all areas.

- Place your hand inside the shoe and press it onto white paper in a motion that mimics the motion that would be made if the shoe were being walked in (i.e., heel to toe).

- Photograph the inked shoe print.

- Document any observed individual or class characteristics.